MineGuard – Instruction Manual

IMPORTANT SAFETY NOTES

- Do not expose devices to water or excessive moisture

- Do not charge devices in explosive/flammable environments

- Do not modify or open the device casings

- Do not ignore alerts—always move to safety first

- Replace devices immediately if damaged or malfunctioning

- Report any technical issues to maintenance team promptly

- Battery Safety: Use only provided chargers; do not attempt battery replacement

Pre-Shift Preparation

- Connect the WristBand and BackPatch to their respective chargers using the provided USB cables.

- Charging indicators: Red LED: Charging in progress Green LED: Fully charged

Wearing & Activating the Device

- Wear the WristBand on either hand (worker's preference).

- Critical: Position the MAX30102 sensor directly over the radial artery on the wrist.

- Wear the BackPatch on the upper back (between shoulder blades).

- Ensure the gas sensor vents are not obstructed by clothing.

- Press power button to activate; device starts automatic monitoring.

Control room handling

- Control ESP receives emergency and dashboard pops an urgent alert (visual + audible).

- Dashboard shows miner ID, sensor values, timestamp, and location (if GPS present).

- Notify mine supervisor and field rescue team.

- Broadcast alert to all miners (control ESP → repeaters → miners): global warning.

- Initiate evacuation if dangerous gas level widespread.

Alert Handling

Local Alert (Miner Device)

- Triggered by high gas levels or abnormal health readings.

- Device activates buzzer + flashing LED immediately.

Network Alert (Repeater & Control)

- Miner device sends an emergency packet to repeater → control ESP.

- Control room receives alert and broadcasts warning to all miners.

- Dashboard highlights the affected miner with real-time values.

Maintenance

- Clean sensor surfaces after each shift

- Verify battery charge levels

- Verify ESP-NOW connectivity in all mine areas

- Review and update miner registration on dashboard

This manual ensures safe, efficient operation of your mining safety monitoring system.